SMART INDIA HACKATHON (SIH) — OFFICIAL TECHNICAL DOSSIER

SIH ID: SIH 26001
LandslideNEI: AI-Driven Operational Landslide Risk Prediction & Real-Time Early-Warning System for North-East India

Complete System Specification, Empirical Earth Observation Datasets, Machine Learning Architecture, Dynamic Multi-Window Rainfall Physics, Geotechnical Factor of Safety, and Unified EOC Command Workstation

| Problem Statement ID | SIH 26001 (Smart India Hackathon) |
| --- | --- |
| Project Codename | LandslideNEI (Operational Landslide Early Warning Engine) |
| Operational Jurisdiction | North-Eastern India (NER) — 8 States (Arunachal Pradesh, Assam, Manipur, Meghalaya, Mizoram, Nagaland, Sikkim, Tripura) |
| Core Technology Stack | Scikit-Learn (Random Forest Model A), Copernicus GLO-30 DEM, ISRIC SoilGrids v2.0, ESA WorldCover, CWC Telemetry, IMD Macro Grids, FastAPI REST API, Three.js 3D WebGL, Leaflet Web GIS, Native Desktop App (Edge WebView2) |
| Empirical Validation Status | 253/253 Pytest Passed ¦ 17/17 Failure/Edge Scenarios Passed ¦ 8/8 State Capitals Verified Live ¦ Desktop EXE Operational Workflow Verified |
| Document Classification | Master Technical Architecture, Working Algorithms, and Operational Dossier (v1.0.0) |

# 1. Executive Summary & Problem Scope (SIH 26001)

Northeast India (NER) comprises eight ecologically fragile and tectonically active states characterized by steep topography, intense monsoonal precipitation, high seismic vulnerability, and rapidly expanding anthropogenic infrastructure (e.g., highway expansion corridors like NH-29 Kohima–Dimapur and NH-10 Teesta Gorge). Under Smart India Hackathon Problem Statement SIH 26001, the LandslideNEI project was engineered to deliver a fully operational, end-to-end, multi-tier early warning system combining static terrain susceptibility with dynamic meteorological telemetry and physics-based geotechnical stability models.

■ CORE SCIENTIFIC & OPERATIONAL PRINCIPLES (SIH 26001) 1. Static Susceptibility != Event-Time Warning: High static terrain predisposition does not indicate slope failure is currently occurring without triggering dynamic rainfall. 2. Zero Synthetic / Fake Data Fallback: Missing observations remain strictly unobserved (None); sensor gaps are NEVER converted into artificial zeroes (0.0 mm). 3. Strict Spatial Telemetry Cap: CWC stations beyond 50.0 km radius are strictly rejected from operational rainfall and flagged as NO_RELIABLE_LOCAL_DATA. 4. Operational Fusion Score is Not a Calibrated Probability: The operational score is an auditable engineering synthesis metric used for tactical ordering, not an empirical percentage probability.

# 2. Complete System Architecture & Operational Topology

LandslideNEI is constructed on a decoupled, resilient microservices architecture specifically hardened for field deployment in intermittent-connectivity disaster environments (State and District Emergency Operations Centers - SEOCs / DEOCs). The end-to-end operational pipeline flows through six cohesive layers:

| Layer | Component / Engine | Operational Responsibility & Contract |
| --- | --- | --- |
| Layer 1: Earth Observation & Telemetry | Raster Ingest & Sensor Pipelines | Windowed extraction from Copernicus GLO-30 DEM (30m), ISRIC SoilGrids v2.0 (250m), ESA WorldCover (10m), CWC telemetry & IMD macro grids. |
| Layer 2: Static Profiler | Model A Random Forest Engine | Extracts elevation, slope, aspect, relief std, soil properties, and land cover to generate baseline static susceptibility score [0.0, 1.0] and tier (LOW, MODERATE, HIGH, VERY_HIGH). |
| Layer 3: Dynamic Rainfall Provider | Multi-Window Hydrometeorology Matcher | Computes Haversine distances to CWC stations (50km cap, 6h freshness limit). Computes 1h, 24h, 3d, 7d accumulations. Implements live Open-Meteo realtime query with 3-retry fallback. |
| Layer 4: Dynamic Trigger Engine | Rule-Based Meteorological Evaluator | Applies engineering thresholds (1h: 20/40mm; 24h: 50/100mm; 3d: 100/200mm; 7d: 150/300mm) to derive dynamic trigger state (NORMAL, WATCH, HIGH, NO_DATA) and continuous score. |
| Layer 5: Risk Fusion Layer | Deterministic 4x4 Decision Matrix | Synthesizes static susceptibility and dynamic trigger into Authoritative Operational Risk Tier (LOW, WATCH, HIGH, CRITICAL) and operational_fusion_score. |
| Layer 6: Presentation & Tactical Command | FastAPI + Web GIS + Windows EXE | Exposes 15 REST endpoints, serves 2D Leaflet interactive map, 3D WebGL Three.js terrain mesh, and embeds in zero-install native Windows desktop app. |

# 3. Authoritative Datasets & Earth Observation (EO) Integration

All data ingest pipelines in LandslideNEI utilize authoritative, globally and nationally validated Earth Observation datasets. No synthetic elevation, fake soil attributes, or fabricated precipitation values exist anywhere in the runtime.

| Dataset / Source | Spatial / Temporal Resolution | Key Variables Ingested | Operational Function in LandslideNEI |
| --- | --- | --- | --- |
| Copernicus GLO-30 DEM | 30-meter ground resolution (COG GeoTIFF) | Elevation (m), Slope (deg), Aspect (deg), 5x5 Relief Std Dev (m) | Primary physical terrain baseline; drives topographic feature extraction and 3D mesh generation. |
| ISRIC SoilGrids v2.0 | 250-meter depth-averaged (0–5 cm) | Clay %, Sand %, Silt %, Bulk Density (kg/dm³), WRB Soil Group | Provides soil geotechnical matrix for shear strength, cohesion, permeability, and liquefaction. |
| ESA WorldCover v200 | 10-meter ground resolution (Sentinel-1/2) | LULC categorical class (Trees, Shrubs, Grass, Crops, Urban, Water) | Captures vegetative root reinforcement and surface land use modification. |
| GSI Landslide Inventory | Historical polygon & point occurrences | 1:1 balanced presence/absence sampling (2014 & 2021) | Supervised machine learning ground truth for Model A Random Forest training. |
| Verified 75 Critical Events Layer | Coordinate-accurate point event database | Coordinates, date, causal mechanism, road corridor, impact metrics | Geospatial contextual overlay on tactical maps for rapid risk validation against known failure zones. |
| CWC Hydro-Meteorology Network | Hourly observation telemetry | 1-hour, 24-hour, 3-day (72h), 7-day rainfall accumulation | Operational in-situ rainfall monitoring (enforcing 50km max distance and 6h freshness window). |
| IMD District & State Macro Grids | Daily district and state meteorological tables | Daily actual (mm), normal (mm), departure percentage (%) | Macro-scale meteorological contextual layer explicitly labeled to prevent false point-scale claims. |

# 4. Complete Technical Stack & Core Competencies

The implementation of LandslideNEI represents an integrated synthesis of data science, geospatial software engineering, geotechnical physics, and full-stack high-performance application development:

| Machine Learning & Statistics | Scikit-Learn (Random Forest Classifier Model A, Pipeline, ColumnTransformer, OneHotEncoder, SimpleImputer), Imbalanced-learn, XGBoost/LightGBM benchmarking, Cross-validation (Stratified 5-Fold), ROC-AUC / Precision-Recall evaluation, Joblib serialization. |
| --- | --- |
| Geospatial & Remote Sensing (GIS) | Rasterio (windowed COG reading, affine geo-transformations, bilinear interpolation), Shapely (Point-in-Polygon containment, prepared geometries, unary union), PyProj (dynamic coordinate transformations: Homolosine IGH to WGS84 EPSG:4326), GADM administrative boundary geometries. |
| Backend & REST Microservices | FastAPI (High-performance asynchronous REST API), Starlette, Pydantic v2 (Strict request/response data contracts, custom field validators), Uvicorn ASGI engine, Python threading & socket management, HTTP connection pools. |
| Tactical Web GIS & Frontend | Vanilla ES6+ JavaScript (Zero heavyweight framework bloat, high responsiveness), Three.js (WebGL hardware-accelerated 3D terrain rendering, dynamic normal mapping, hypsometric shader tinting), Leaflet.js (Interactive geospatial multi-layer mapping), HTML5, Modern CSS (Glassmorphism, Dark-mode, Material Symbols). |
| Desktop Systems Engineering | PyInstaller (Standalone single-executable & folder bundling), Native Windows Edge WebView2 App-Mode execution (`msedge.exe --app=...`), Windows Kernel32 API integration (`AttachConsole`, `GetStdHandle`, `open_osfhandle`), Port auto-negotiation. |
| DevOps, Reliability & Quality Assurance | Pytest automated test runner (253 tests across 15 test suites), Pytest-cov, Fault-tolerant degraded modes, In-memory raster caching (spatial LRU), Vectorized Haversine algorithms, PEP-8 compliance. |

# 5. Working Algorithms & Mathematical Formulations

## 5.1 Algorithm 1: Static Susceptibility Modeling (Random Forest Model A)

Static landslide susceptibility is modeled through Model A—an environmental-only ensemble Random Forest classifier trained on 1:1 balanced historical failure inventories from the Geological Survey of India (GSI). The model maps 11 physical conditioning features into a continuous static predisposition score:

Feature Vector x = [Elevation (z), Slope (β), Aspect (α), Relief_Std_5x5 (σ), Soil_WRB_Class, Clay_%, Sand_%, Silt_%, Bulk_Density (ρ), LULC_Class]

Mathematical Ensemble Formulation:
For an ensemble of M decision trees (M = 100), the static susceptibility score is given by:
   S_static(x) = (1 / M) * Σ_{m=1}^{M} P_m(Y = 1 | x)

Operational Susceptibility Classification:
• LOW:          0.00 <= S_static < 0.25 (Stable plains and valley bottoms)
• MODERATE:     0.25 <= S_static < 0.50 (Gentle rolling terrain, moderate cohesion)
• HIGH:         0.50 <= S_static < 0.75 (Steep escarpments, weathered soils)
• VERY_HIGH:    0.75 <= S_static <= 1.00 (Critical mountain ridges, high relief, fragile regolith)

## 5.2 Algorithm 2: Dynamic Multi-Window Rainfall Trigger Engine

Rainfall is the primary dynamic landslide trigger across Northeast India. The engine continuously evaluates accumulated rainfall over four operational windows: 1-hour (flash / burst trigger), 24-hour (daily saturation), 3-day (antecedent moisture buildup), and 7-day (deep regolith saturation).

Configurable Operational Engineering Thresholds:
• 1-Hour Window (1h):   WATCH = 20.0 mm | HIGH = 40.0 mm
• 24-Hour Window (24h):  WATCH = 50.0 mm | HIGH = 100.0 mm
• 3-Day Window (72h):   WATCH = 100.0 mm | HIGH = 200.0 mm
• 7-Day Window (7d):    WATCH = 150.0 mm | HIGH = 300.0 mm

Intensity Ratio Formulation:
For each observed window w in {1h, 24h, 3d, 7d}, the relative intensity ratio is:
   R_w = P_w / Θ_{w, HIGH}

Trigger Score T_score Formulation in [0.0, 1.0]:
• If HIGH trigger breached (any P_w >= Θ_{w, HIGH}):
   T_score = min(1.0, 0.70 + min(0.30, (max(R_w) - 1.0) * 0.15))
• If WATCH trigger breached (any P_w >= Θ_{w, WATCH}):
   T_score = 0.40 + min(0.29, (max(R_w) - 0.50) * 0.58)
• If NORMAL (all P_w < Θ_{w, WATCH}):
   T_score = min(0.39, max(R_w) * 0.78)
• If NO_DATA (sensors offline or >50km away):
   T_score = None, Trigger_Level = NO_DATA

## 5.3 Algorithm 3: Deterministic Static-Dynamic Risk Fusion Matrix

To guarantee 100% auditable, deterministic operational decision-making, LandslideNEI synthesizes static terrain susceptibility and dynamic rainfall triggers through an explicit 4x4 matrix without black-box ML blending:

| Static Susceptibility | Rain: NORMAL | Rain: WATCH | Rain: HIGH | Rain: NO_DATA |
| --- | --- | --- | --- | --- |
| LOW | LOW | WATCH | WATCH | LOW |
| MODERATE | LOW | WATCH | HIGH | WATCH |
| HIGH | WATCH | HIGH | CRITICAL | WATCH |
| VERY_HIGH | WATCH | HIGH | CRITICAL | HIGH |

Composite Operational Fusion Score (operational_fusion_score):
• When dynamic rainfall telemetry is active:
   operational_fusion_score = round(min(1.0, max(0.0, (0.5 * S_static) + (0.5 * T_score))), 4)
   Mode: DUAL_LAYER_WEIGHTED_SYNTHESIS
• When dynamic rainfall is unobserved / offline (NO_DATA):
   operational_fusion_score = round(S_static, 4)
   Mode: STATIC_BASELINE_ONLY_RAINFALL_UNOBSERVED

Crucial Semantic Rule: The categorical risk_level (LOW/WATCH/HIGH/CRITICAL) is the authoritative operational decision. The numerical score is an ordering and visualization metric, never described as a calibrated probability.

## 5.4 Algorithm 4: Geotechnical Factor of Safety (FS) & Anthropogenic Surcharge

For slope-scale geotechnical diagnostics, LandslideNEI incorporates an infinite-slope limit-equilibrium stability formulation enhanced with vegetative root cohesion (c_r), pore-water pressure surcharge (u), and anthropogenic highway excavation destabilization (τ_anthro):

   FS = [ c' + c_r + (γ_m * z * cos²(β) - u) * tan(φ') ] / [ γ_m * z * sin(β) * cos(β) + τ_anthro ]

Parameters:
• c': Effective soil cohesion (kPa), derived from clay/silt fraction and soil taxonomy.
• c_r: Root reinforcement cohesion (kPa), calculated dynamically from ESA WorldCover tree canopy density.
• γ_m: Moist unit weight of soil regolith (~18.5 kN/m³).
• z: Estimated slip surface depth (~2.5 m).
• β: Local topographic slope angle (degrees) derived from Copernicus GLO-30 DEM.
• φ': Internal angle of friction (degrees), parameterized by soil sand content.
• u: Regolith pore-water pressure (kPa), derived from moisture saturation ratio S_r = θ / n.
• τ_anthro: Destabilizing shear surcharge resulting from unsupported road cuts, steep toe excavations, or blocked drainage.

Engineering Verdicts:
• FS > 1.30: STABLE (Adequate safety margin)
• 1.00 <= FS <= 1.30: MARGINALLY_STABLE (Prone to failure under intense precipitation)
• FS < 1.00: UNSTABLE (Active failure state / immediate landslide hazard)

## 5.5 Algorithm 5: Micro-Topography & Morphometric Curvature Extraction

To detect micro-relief failure bowls and convergent gullying at village resolution, a 3x3 second-order polynomial surface is fitted using the Zevenbergen-Thorne / Evans algorithm:
   Z = a*x² + b*y² + c*x*y + d*x + e*y + f

Derived Morphometric Indices:
• Profile Curvature (k_prof): Measures downslope flow acceleration/deceleration. Strongly negative values indicate concave depositional hollows.
• Planform Curvature (k_plan): Measures lateral flow convergence/divergence. Negative values indicate flow convergence into slope gullies.
• Topographic Position Index (TPI): TPI = Z_0 - Z_mean(5x5). Categorizes slope into valley bottoms, mid-slope spurs, or exposed ridges.
• Topographic Wetness Index (TWI): TWI = ln(a / tan(β)), identifying moisture accumulation zones.

## 5.6 Algorithm 6: Vectorized Haversine Telemetry Matching & Multi-Tier Fallback

When an inference request arrives for coordinates (lat, lon), the provider computes great-circle distances across all CWC stations using vectorized NumPy Haversine:
   d = 2R * arcsin( sqrt( sin²(Δφ/2) + cos(φ1)*cos(φ2)*sin²(Δλ/2) ) )

Multi-Tier Resiliency Workflow:
1. Distance Filter: If nearest station distance d > 50.0 km, local CWC data is strictly marked NO_LOCAL_DATA. Missing data remains None.
2. Freshness Filter: If telemetry age > 6.0 hours, data is labeled STALE, triggering advisory alerts.
3. Live Real-Time Integration: When auto_refetch=True, system executes 3 consecutive real-time queries to Open-Meteo API. If any succeed, live observation is used.
4. Offline Degraded State: If network is offline, gracefully engages IMD district context, and if unavailable, enters unobserved mode preserving static ML.

## 5.7 Algorithm 7: 3D DEM Adaptive Mesh Decimation & WebGL Visualization

For interactive 3D tactical terrain exploration, the engine extracts a 128x128 or 256x256 regular elevation grid from Copernicus GLO-30 rasters. The server returns normalized elevation buffers, bounding box coordinates, and hypsometric relief stats. In the browser/workstation, Three.js constructs a WebGL PlaneGeometry, maps dynamic vertex normals, and applies risk-tier color contouring (Green/Amber/Red) in real-time.

# 6. Unified REST API Specification & Microservices Contract

The LandslideNEI FastAPI microservices suite exposes 15 dedicated endpoints covering inference, location profiling, 3D terrain geometry, historical layers, and operational health:

| Method | Endpoint | Purpose | Data Source | Status |
| --- | --- | --- | --- | --- |
| GET | /health | System health check & engine readiness | Internal State | 200 OK (9.3 ms) |
| GET | /api/v1/health | Versioned health & provider status | Internal State | 200 OK (9.4 ms) |
| GET | /api/v1/info | API metadata, bounding box & version | Config API | 200 OK (1.2 ms) |
| GET | /model-info | ML Model A architecture & features | Model Metadata | 200 OK (1.5 ms) |
| POST | /api/v1/predict | Unified operational landslide prediction | Model A + CWC + IMD | 200 OK (107 ms) |
| POST | /api/v1/profile | Precision terrain & soil profiling | Copernicus + SoilGrids | 200 OK (119 ms) |
| POST | /api/v1/rainfall/refetch | Force live regional rainfall telemetry | Open-Meteo Live API | 200 OK (380 ms) |
| GET | /api/v1/terrain/metadata | Copernicus DEM coverage & tiles | DEM Manifest | 200 OK (12.6 ms) |
| GET | /api/v1/terrain/mesh | 3D WebGL elevation mesh extraction | Copernicus GLO-30 | 200 OK (45 ms) |
| GET | /api/v1/layers/status | Layer operational status check | System Registry | 200 OK (2.1 ms) |
| GET | /api/v1/layers/historical-landslides | 75 verified historical landslide events | Verified GSI Layer | 200 OK (15.8 ms) |
| GET | / | API root landing & documentation router | FastAPI Core | 200 OK (1.0 ms) |
| GET | /dashboard/ | Tactical EOC GIS command dashboard | Static HTML/JS/CSS | 200 OK (Static) |
| GET | /website/ | Public product information portal | Static HTML/JS/CSS | 200 OK (Static) |
| GET | /download/windows | Desktop EXE distribution downloader | Dist Artifact | 200 OK (Binary) |

# 7. Emergency Operations Center (EOC) Command Dashboard & Desktop Application

To ensure seamless field operation by disaster managers, district collectors, and geotechnical engineers, LandslideNEI is deployed in two modalities:

1. Tactical Web EOC Dashboard (Browser / Network Access):
• Dual 2D/3D Tactical Mapping: Seamless toggle between 2D Leaflet spatial overview and Three.js 3D WebGL terrain exploration.
• Live Telemetry HUD: Real-time UTC clock, elevation, coordinates, operational verdict banner, and animated SVG needle gauge.
• Geotechnical & Micro-Climate Panels: Displays slope severity, plan/profile curvature, soil bulk density, clay fraction, and rainfall windows.
• Historical Ground-Truth Verification: Interactive toggle displaying 75 verified critical landslide events with incident metadata.

2. Standalone Windows Desktop Workstation (LANDSLIDENEI.exe):
• Built with PyInstaller and embedded local Uvicorn ASGI server.
• Auto-launches Microsoft Edge in dedicated windowed application mode (`--app=http://127.0.0.1:PORT/dashboard/`).
• Native Windows integration: Automatically attaches to parent console (`AttachConsole`), handles redirected STDIO, and auto-negotiates free ports.
• Self-contained offline capability: Fully executes Model A inference, GLO-30 terrain queries, and risk fusion with zero internet connection.

# 8. Empirical Verification Across All 8 Northeast India States

The system was rigorously audited and executed live across all 8 NER state capitals and critical transport corridors. The following empirical results were recorded from direct API execution:

| State Capital / Corridor | State | Coordinates | Static Susc. | Nearest CWC Station (Distance) | Operational Risk |
| --- | --- | --- | --- | --- | --- |
| Kohima (NH-29) | Nagaland | 25.6740°N, 94.1120°E | MODERATE (0.4595) | Bokajan (50.16 km — Beyond 50km Cap -> NO_DATA) | WATCH (0.4595) |
| Shillong Peak | Meghalaya | 25.5788°N, 91.8933°E | MODERATE (0.3995) | Guwahati DC Court (70.11 km — Beyond 50km Cap -> NO_DATA) | WATCH (0.3995) |
| Tezpur Sonitpur | Assam | 26.6528°N, 92.7926°E | LOW (0.1571) | Tezpur (4.13 km — In-Situ Telemetry Connected) | LOW (0.0785) |
| Namchi Ridge | Sikkim | 27.1664°N, 88.3639°E | VERY_HIGH (0.8409) | Majitar (7.77 km — In-Situ Telemetry Connected) | WATCH (0.4302) |
| Imphal Valley | Manipur | 24.8170°N, 93.9368°E | LOW (0.1105) | Amraghat (26.16 km — In-Situ Telemetry Connected) | LOW (0.0747) |
| Lunglei Ridge | Mizoram | 22.8878°N, 92.7397°E | HIGH (0.7433) | GUMTI HYDRO (111.45 km — Beyond 50km Cap -> NO_DATA) | WATCH (0.7433) |
| Agartala Urban | Tripura | 23.8315°N, 91.2868°E | LOW (0.2441) | Sonamura (40.02 km — In-Situ Telemetry Connected) | LOW (0.1270) |
| Itanagar Foothills | Arunachal | 27.0844°N, 93.6053°E | HIGH (0.6347) | Badatighat (38.43 km — In-Situ Telemetry Connected) | WATCH (0.3271) |

# 9. Edge-Case Resiliency, Security & Degraded Offline Mode

A critical mandate for disaster-response software is fault tolerance under extreme conditions. LandslideNEI was subjected to a comprehensive 17-scenario edge-case test battery. All 17 passed without a single unhandled exception or crash:

| Case # | Failure / Boundary Scenario | Expected Handling & HTTP Code | Verified Test Result |
| --- | --- | --- | --- |
| 01 | Valid Coordinates (Kohima) | 200 OK with full susceptibility and risk payload | PASS — Risk level WATCH generated |
| 02 | Coordinates Outside NER Domain (New Delhi) | 400 Bad Request with OUTSIDE_SUPPORTED_DOMAIN error | PASS — Rejected cleanly by GADM polygon |
| 03 | Invalid Latitude (lat > 90°) | 422 Unprocessable Content schema validation error | PASS — Rejected by Pydantic v2 |
| 04 | Invalid Longitude (lon > 180°) | 422 Unprocessable Content schema validation error | PASS — Rejected by Pydantic v2 |
| 05 | Missing Parameters (Latitude omitted) | 422 Unprocessable Content field required error | PASS — Field required exception caught |
| 06 | Null / None Coordinates | 422 Unprocessable Content valid number required error | PASS — Rejected cleanly |
| 07 | Extreme Coordinates (-89.9°, -179.9°) | 400 Bad Request domain guardrail | PASS — Rejected by spatial bounds |
| 08 | No CWC Station within 50 km (Lunglei) | 200 OK, rainfall status NO_RELIABLE_LOCAL_STATION | PASS — GUMTI 111km rejected, rainfall=None |
| 09 | Stale CWC Data (>6 hours old) | 200 OK, marked STALE, triggers advisory reasons | PASS — Correctly tagged with age 26601h |
| 10 | Missing Rainfall Window | 200 OK, window remains None (NOT converted to 0.0) | PASS — Verified None != 0.0 mm |
| 11 | IMD Gridded Data Unavailable | 200 OK, falls back to CWC without macro enrichment | PASS — Gracefully handled without crash |
| 12 | CWC Hydro-Telemetry Unavailable | 200 OK, enters unobserved mode (Trigger: NO_DATA) | PASS — Static ML preserved bit-for-bit |
| 13 | Soil Raster Tile Unavailable | 200 OK, scikit-learn median imputation engages | PASS — Zero failure in feature pipeline |
| 14 | DEM Raster Unavailable | Safe handling via focal terrain cache or fallback | PASS — No unhandled process termination |
| 15 | Malformed JSON Request Body | 422 / 400 Bad Request with structured error JSON | PASS — Rejected before entering business logic |
| 16 | Upstream Real-time API Timeout | 200 OK, 3 retries engage, falls back to offline | PASS — Handled via fallback_engaged=True |
| 17 | Upstream HTTP 500 Failure | 200 OK, resilient fallback to offline cache | PASS — Handled via fallback_engaged=True |

# 10. Conclusion, Innovation Highlights & Deployment Roadmap

LandslideNEI successfully addresses all foundational objectives of Smart India Hackathon Problem Statement SIH 26001. By enforcing strict scientific honesty (separating static predisposition from dynamic triggers, disallowing fake zeroes, and enforcing a 50km spatial telemetry cap), the platform establishes a reliable, mission-critical foundation for landslide disaster risk reduction in Northeast India.

Key Architectural Innovations:
1. True Dual-Layer Fusion: Deterministic 4x4 matrix eliminates black-box AI opacity during emergency operations.
2. Zero-Dependency Tactical GIS: WebGL 3D terrain exploration running smoothly even on basic field laptops without specialized GIS workstations.
3. Multi-Tier Telemetry Resilience: 3-attempt live real-time API loop with automated fallback to in-situ CWC and IMD macro tables.
4. Fully Self-Contained Desktop EXE: Packaged single-executable operational workstation with pre-warmed inference engines and zero external runtime dependencies.

Immediate Deployment Roadmap:
• Phase 1 (Q4 2026): Pilot installation across Nagaland (Kohima EOC) and Sikkim (Gangtok/Namchi EOC).
• Phase 2 (Q1 2027): Integration of National Remote Sensing Centre (NRSC) SAR interferometry / InSAR surface displacement alerts.
• Phase 3 (Q2 2027): Direct integration with NDMA / SDMA Common Alerting Protocol (CAP) for automated SMS warning broadcasts to highway travelers.

SUBMITTED FOR SMART INDIA HACKATHON EVALUATION
PROBLEM STATEMENT ID: SIH 26001
TEAM LANDSLIDENEI — ALL SYSTEMS OPERATIONAL